Accelerometer Test Module                                        QK-05-034

Description

The Accelerometer Test Module is designed to measure acceleration in three dimensions.  It contains a Raspberry P1 microprocessor RP2040 coupled to an accelerometer LSM303C.  The module contains an 80 mAH battery, USB-C connectivity to a PC plus a push button and LEDs connected to the RP2040

Specification

Operating Temperature 	0 to 50 deg C

Interface	USB - C

Voltage				5 volts via the USB- C connector regulated to 3.3 volts on   					board

Current				Typical 22 mA

Size				26 x 60 mm

Weight				8.5g – approx.

Microcontroller			RP2040

Memory			3 MB – Typical AT25SF321

Accelerometer			LSM303   3 axis accelerometer / magnetometer

Power Management		TP4056 – AD3401

Battery				LIR2032 Li-Ion 3.6/3.8 volts nominal 80mAH

Data Transfer LED		Blue LED to indicate data transfer via the USB-C when using 					the Kookaberry Firmware

Soft Reset Push Button		When used with the Kookaberry Firmware

Push Button			For use with applications

LEDs				Red and Orange LEDs for use with applications

Application

The module can be used to measure acceleration in three dimensions

Module Description

The module has been designed as a subset of the Kookaberry and as such can run the Kookaberry firmware and make use of the IDE features in the KookaSuite -  being the KookaIDE, KookaBlockly and the KookaTW.  The latest version of KookaSuite can be downloaded from Github at

https://github.com/kookaberry/kooka-releases/tree/master/KookaSuite

The RP2040 microcontroller is connected to the USB-C and is supported by 3MB of memory and a 12MHz crystal. It operates at 3.2 volts

The on board LIR2032 3.6 volt Li-Ion battery is charged via  Vbus (5 volts) from the USB-C connector and the Power Management circuit which also regulates the module voltage VCC to 3.2 volts

Two jumpers (JP2 and JP3) allow the battery to be connected to the module.  With the jumper in the J2 position the battery is not connected to the module.  With the jumper in the J3 position the battery is connected, it can be charged via the USB-C and it can also supply power to the board if the USB-C is not connected.

A green LED shows when power is connected to the board.  When the board is connected via the USB-C connector to a PC and the battery is connected via jumper J3 the

- green LED will show power

- the Power Management blue LED will show charging of the battery and

- the orange LED will show when the battery is charged

The LSM303 is an accelerometer / Magnetometer integrated circuit.  It is connected to the RP2040 via an I2C bus and provides magnetic and acceleration readings in three dimensions

The Soft Reset button is a feature of the Kookaberry firmware and when pushed provides a software reset to the Kookaberry firmware./ applications

There are two user accessible LEDs (Red and Orange) plus a push button

Mapping

Sample Code - Using KookaBlockly and KookaIDE

KookaBlockly Script to Flash the Red and Orange LEDs

KookaIDE - MicroPython Script to Flash Red and Orange LEDs

KookaBlockly Script to test Push Button

KookaIDE - MicroPython Script to test Push Button

KookaBlockly Script to test LSM303 Acceleration

KookaIDE - MicroPython Script to test LSM303 Acceleration

Typical Display